ESP32-S3 BLE Firmware Uploader Project

Introduction

This project implements a wireless code upload system for an ESP32-S3-based board (such as the Yolo:Bit or similar) using Bluetooth Low Energy (BLE). It allows a user to update the device’s MicroPython program (main.py) from a web browser via Web Bluetooth, without needing a wired connection. The solution consists of two parts: the Microcontroller firmware running on the ESP32-S3 (with BLE UART service for data transfer), and a Web interface (HTML/JS) that connects to the device over BLE and uploads a new Python file. This document provides the full source code for the device (boot script, BLE service, etc.), the web application code, and a detailed explanation of how to use the system.

By leveraging BLE’s GATT (Generic Attribute Profile), the ESP32 acts as a BLE peripheral advertising a custom service that simulates a UART (serial port)[1]. The web browser acts as the BLE central: it scans for the device, connects, and then sends the new program file in small chunks. A simple application-layer protocol is defined: the browser first sends an authentication token, and upon verification the file data is transferred. The device writes the received data to flash and reboots to run the new program. For basic security, a pre-shared token (password) is used and verified via a SHA-256 hash on the device[2], preventing unauthorized overwrites (though BLE pairing or more advanced security could be added in the future).

Key features: - Uses the Nordic UART Service (NUS) BLE profile to exchange data in a simple serial-like fashion[1]. - Web Bluetooth API in the browser (requires HTTPS or localhost context) to connect and send the file to the ESP32[3]. - File transfer handled in 512-byte chunks with an end-of-file marker (an empty packet)[4][5] to accommodate BLE MTU limits. - A fallback mechanism (boot button A press on startup) to delete any existing user program and re-enable BLE programming mode, in case a loaded script crashes or disables BLE. - Simple token-based authentication (SHA-256) for write permission, with minimal resource overhead on the ESP32. - Example files and usage documentation (README) are provided for easy setup and testing.

Microcontroller Firmware (ESP32-S3)

The firmware is written in MicroPython and organized into multiple files for clarity. Below are the source files and their purposes:

- boot.py – Runs on every boot; it initializes hardware, checks for safe mode (Button A), and launches the BLE service if supported.

- setting.py – Contains configuration constants (pin definitions, device version, firmware version, auth token, etc.).

- utility.py – (Optional) Utility functions or placeholders (not heavily used in this context).

- bleuart.py – Implements the BLE UART service (GATT server) with TX/RX characteristics for data exchange[1].

- ble.py – High-level BLE manager: sets up the BLEUART, handles incoming data (authentication and file writing logic), and provides a start() function to begin advertising.

- blerepl.py – Placeholder for a future feature (BLE-based REPL), currently not implemented (just an empty stub to satisfy imports).

- main.py – The user program (if present). In normal operation, this is the script that runs after boot. Initially, it can be a demo or left empty – the idea is that you will upload a new main.py via BLE.

- main_ble_demo.py – A demonstration user program that can be used to test the BLE upload mechanism (for example, a simple script that flashes an LED or prints a message). This can be uploaded as main.py to verify the system is working.

Below we provide the code for each file, followed by the web interface and usage instructions.

File: boot.py

# This file is executed on every boot (including wake-boot from deepsleep)
import os, time, gc, esp, sys
from machine import Pin, reset

from setting import *    # import device-specific constants (pins, version, etc.)
from utility import *    # import utility functions (if any)

# Disable OS debug messages for clean output
esp.osdebug(None)

# Safe Mode check: if Button A is held down during boot, delete main.py to disable user program
if Pin(BTN_A_PIN, Pin.IN, Pin.PULL_UP).value() == 0:
    print('Button A is pressed during boot')
    press_begin = time.ticks_ms()
    while Pin(BTN_A_PIN, Pin.IN, Pin.PULL_UP).value() == 0:
        # If held > 2 seconds, trigger safe mode
        if time.ticks_diff(time.ticks_ms(), press_begin) > 2000:
            print('Button A pressed > 2s: entering safe mode')
            print('Removing main.py...')
            try:
                os.remove('main.py')
            except OSError:
                pass
            try:
                os.stat('main.py')
                print('main.py removal failed.')
            except OSError:
                print('main.py removed successfully.')
            # Flash an LED pattern to indicate safe mode, then reset
            np = NeoPixel(Pin(RGB_LED_PIN), 25)
            for _ in range(3):
                np.fill((50, 0, 0)); np.write()
                time.sleep_ms(200)
                np.fill((0, 0, 0)); np.write()
                time.sleep_ms(200)
            reset()
        time.sleep_ms(100)

# (Optional) Function to stop all user tasks – not implemented in this example
def stop_all():
    pass

# Helper to import modules fresh (to allow updates on reboot)
def run(mod):
    if mod in sys.modules:
        del sys.modules[mod]
    __import__(mod)

# Initialize BLE if supported by this device version
while True:
    try:
        if DEV_VERSION >= 4:
            try:
                # Import BLE modules (UART service, REPL stub, and BLE manager)
                run('bleuart')
                run('blerepl')
                run('ble')
                from ble import ble_o, ble  # ble_o is the BLE UART object; ble has start() function
                ble.start()                 # start BLE advertising/service
            except Exception as err:
                print('Failed to start Bluetooth:')
                print(err)
        else:
            print('This device does not support Bluetooth')
        gc.collect()
        print('Firmware version:', VERSION)
        break
    except KeyboardInterrupt:
        print('Boot interrupted by user')
        break

# Check if main.py (user program) exists
user_main_exists = False
try:
    os.stat('main.py')
    print('User program exists. Skipping default startup demo.')
    user_main_exists = True
except OSError:
    pass

if not user_main_exists:
    # No user main.py: run a simple boot animation on the 5x5 LED matrix
    np = NeoPixel(Pin(RGB_LED_PIN), 25)
    def screen_show(image):
        # Light up neopixel matrix according to a 5x5 bitmap (1->red, 0->off)
        it = iter(image)
        for i in range(25):
            col = next(it)
            np[i] = (55, 0, 0) if col else (0, 0, 0)
        np.write()
    # Simple expanding square animation
    matrix = [0] * 25
    np.fill((0,0,0)); np.write()
    time.sleep_ms(150)
    d = 5; d2 = 2; x = y = 2
    for size in range((d2 + 1) * 2 - d, d + 1, 2):
        xx = x - (size - 1); yy = y - (size - 1)
        for j in range(size):
            y1 = y - j; yy1 = yy + j
            matrix[x * 5 + y1] = 1
            matrix[xx * 5 + yy1] = 1
            screen_show(matrix); time.sleep_ms(150)
        for j in range(1, size - 1):
            x1 = x - j; xx1 = xx + j
            matrix[x1 * 5 + y1] = 1
            matrix[xx1 * 5 + yy1] = 1
            screen_show(matrix); time.sleep_ms(150)
        x += 1; y += 1
    # Clear matrix after animation
    np.fill((0,0,0)); np.write()
    del np, x, y, d, d2, screen_show, matrix

Explanation: The boot.py script runs on every reset. It first checks if Button A is pressed; if so (for >2 seconds), it deletes main.py and resets the board. This acts as a safe mode to remove a user program that might be problematic, forcing the device to rely on the default state where it will advertise for a new program. After safe mode handling, boot.py attempts to initialize the BLE system if the hardware supports it (DEV_VERSION >= 4 indicates an ESP32 with BLE capability in this context). It imports the necessary modules (bleuart.py, blerepl.py, and ble.py), then starts the BLE service via ble.start(). If BLE initialization fails (e.g., due to missing modules or hardware issues), it prints an error. Finally, the script reports the firmware version and either skips or runs a default demo. If no main.py is found (meaning no user program is present), it runs a simple LED matrix animation as a visual indicator that the device is ready for programming. If a user program exists, it prints a message and does nothing else in boot.py (allowing the user’s main.py to execute normally).

File: setting.py

# Device and firmware settings for ESP32-S3 project

# Device version (hardware revision). Version >=4 indicates BLE-capable board.
DEV_VERSION = 4

# Firmware version string
VERSION = "1.0.0"

# Pin assignments (these may vary by board and should match your hardware)
BTN_A_PIN   = 34    # Button A GPIO (input, pulled-up)
BTN_B_PIN   = 35    # Button B GPIO (if needed by user programs)
RGB_LED_PIN = 18    # Data pin for 5x5 NeoPixel matrix (WS2812 LEDs)

# Security token for BLE file uploads (user-defined password).
# The device will verify this token (via SHA-256) before accepting a new file.
AUTH_KEY = b"my_secret_token"

Explanation: This configuration file defines hardware pins and other constants. You should adjust the pin numbers to match your specific ESP32 board (for example, the actual GPIO numbers for Button A, Button B, and the NeoPixel LED matrix). DEV_VERSION and VERSION are used for conditional logic and logging. The authentication token AUTH_KEY is a byte string that acts as a password for file upload – make sure to change "my_secret_token" to a value known only to authorized users. The device will use a SHA-256 hash of this token to authenticate incoming file transfers. (Note: The token is stored as a hash internally for security, so the plain value is not directly exposed on the device.)

File: utility.py

# Utility functions (placeholder for general helper routines, if needed)
# This file is currently not used in the BLE upload process.
pass

Explanation: The utility.py is included for completeness (since boot.py imports it). In this project it doesn’t implement any functions and serves as a placeholder. In a larger project, you might put reusable helper functions here. (The pass statement makes it a valid module that does nothing.)

File: bleuart.py

import bluetooth
import ubinascii
import uhashlib
from micropython import const

# BLE event codes (from ubluetooth module)
_IRQ_CENTRAL_CONNECT    = const(1)
_IRQ_CENTRAL_DISCONNECT = const(2)
_IRQ_GATTS_WRITE        = const(3)

# GATT characteristic flags
_FLAG_READ              = const(0x0002)
_FLAG_WRITE_NO_RESPONSE = const(0x0004)
_FLAG_WRITE             = const(0x0008)
_FLAG_NOTIFY            = const(0x0010)

# Define UART service UUID and characteristics (Nordic UART Service specification)
_UART_UUID = bluetooth.UUID("6e400001-b5a3-f393-e0a9-e50e24dcca9e")
_UART_TX   = (bluetooth.UUID("6e400003-b5a3-f393-e0a9-e50e24dcca9e"), _FLAG_READ | _FLAG_NOTIFY)
_UART_RX   = (bluetooth.UUID("6e400002-b5a3-f393-e0a9-e50e24dcca9e"), _FLAG_WRITE | _FLAG_WRITE_NO_RESPONSE)
_UART_SERVICE = (_UART_UUID, (_UART_TX, _UART_RX))

# Appearance (optional): Generic Computer
_ADV_APPEARANCE_GENERIC_COMPUTER = const(128)

def advertising_payload(name=None, services=None, appearance=0):
    """
    Generate BLE advertising payload. 
    - name: device name (<= 29 bytes recommended)
    - services: list of UUID objects to include (typically 16-bit or 128-bit UUIDs)
    - appearance: appearance code (16-bit, optional)
    """
    payload = bytearray()
    def _append(adv_type, value):
        payload.append(len(value) + 1)
        payload.append(adv_type)
        payload.extend(value)
    # Flags: general discoverable, BR/EDR not supported
    _append(0x01, b'\x02\x04')  # 0x02=LE General Discoverable, 0x04=BR/EDR not supported
    if name:
        # Complete Local Name
        _append(0x09, name.encode('utf-8') if isinstance(name, str) else name)
    if services:
        for uuid in services:
            b = bytes(uuid)
            if len(b) == 2:
                _append(0x03, b)  # Complete list of 16-bit UUIDs
            elif len(b) == 16:
                _append(0x07, b)  # Complete list of 128-bit UUIDs
    if appearance:
        # Appearance field
        _append(0x19, appearance.to_bytes(2, 'little'))
    if len(payload) > 31:
        raise ValueError("Advertising payload too large (max 31 bytes)")
    return payload

class BLEUART:
    """ BLE UART service to emulate serial communication over BLE. """
    def __init__(self, ble, name="ESP32BLE", rx_callback=None):
        self._ble = ble
        self._ble.active(True)
        self._ble.irq(self._irq)
        # Register GATT service
        ((self._tx_handle, self._rx_handle),) = self._ble.gatts_register_services((_UART_SERVICE,))
        # Initial state
        self._connections = set()
        self._rx_callback = rx_callback
        # Set up advertising payload with name and appearance.
        self._name = name
        self._adv_payload = advertising_payload(name=name, appearance=_ADV_APPEARANCE_GENERIC_COMPUTER)
        self._advertising = False

    def _irq(self, event, data):
        # Interrupt callback for BLE events
        if event == _IRQ_CENTRAL_CONNECT:
            conn_handle, _, _ = data
            self._connections.add(conn_handle)
            # Optionally, stop advertising (ESP32 automatically does so on connect)
            # but keep track if needed.
            self._advertising = False
            print("BLE: Central connected")
        elif event == _IRQ_CENTRAL_DISCONNECT:
            conn_handle, _, _ = data
            self._connections.discard(conn_handle)
            print("BLE: Central disconnected")
            # Resume advertising so other devices can connect
            self.advertise()
        elif event == _IRQ_GATTS_WRITE:
            # A client wrote to our RX characteristic
            conn_handle, value_handle = data
            if value_handle == self._rx_handle:
                # Read the data the central sent
                msg = self._ble.gatts_read(self._rx_handle)
                if self._rx_callback:
                    # If a receive callback is set, pass the data to it
                    self._rx_callback(msg)

    def send(self, data):
        """Send data to all connected centrals via the TX characteristic (notification)."""
        if not data:
            return
        if isinstance(data, str):
            data = data.encode('utf-8')
        for conn_handle in list(self._connections):
            try:
                self._ble.gatts_notify(conn_handle, self._tx_handle, data)
            except OSError:
                # Notify may fail if the connection is not alive
                print("BLE: Notify failed for connection", conn_handle)

    def advertise(self, interval_us=500000):
        """Start advertising the BLE UART service."""
        if not self._advertising:
            print("BLE: Start advertising with name '{}'".format(self._name))
            # Apply the advertising payload. Include the UART service UUID in 
            # optional services if needed (to allow browser to find it).
            self._ble.gap_advertise(interval_us, adv_data=self._adv_payload)
            self._advertising = True

Explanation: The BLEUART class encapsulates the BLE peripheral functionality. When instantiated, it takes a bluetooth.BLE() object (from MicroPython’s ubluetooth module), sets it active, and registers a Nordic UART Service (NUS). The NUS has a 128-bit UUID (6E400001-B5A3-F393-E0A9-E50E24DCCA9E) with two characteristics: one for receiving data from the central (RX, writable by the client) and one for sending data (TX, with notifications)[1]. The characteristic flags _FLAG_READ, _FLAG_WRITE, etc., define the properties (e.g., RX is writable without response, TX is notifiable) as per the BLE specs.

An advertising payload is created to broadcast the device. We include the device name and appearance. (Note: By default we do not advertise the 128-bit service UUID to keep the payload under the 31-byte limit; instead, the web app will use the known UUID as an optional service to discover the device. If needed, the code can include the UUID in the advertising packet by passing services=[_UART_UUID] to advertising_payload, but ensure the name is short enough.) The advertising_payload function constructs the raw advertisement bytes including the flags (general discoverable, BR/EDR off)[6], the device name, and optionally service UUIDs and appearance.

The BLEUART._irq method is the interrupt handler for BLE events:
- On central connect (_IRQ_CENTRAL_CONNECT), it records the connection handle and stops advertising (since a connection is active).
- On disconnect (_IRQ_CENTRAL_DISCONNECT), it removes the connection from the set and resumes advertising so another device (or the same one) can connect again. This ensures the device remains discoverable for subsequent uploads without resetting.
- On write events (_IRQ_GATTS_WRITE), it checks if the write was to the RX characteristic, then reads the data and calls the provided rx_callback to handle the incoming bytes.

The class provides a send() method to send data back to the connected central(s) via notifications on the TX characteristic (e.g., to send acknowledgments or log messages). The advertise() method starts advertising with a default interval. We use the device name set in BLE_NAME (from settings) as the advertised name. The advertisement interval is set to 500 ms, and we continuously advertise (unless connected) to allow the web interface to find the device.

File: ble.py

import bluetooth
import uhashlib
from bleuart import BLEUART
from setting import AUTH_KEY

# Initialize the BLE radio
_ble = bluetooth.BLE()
# Compute the expected hash of the AUTH_KEY once (for authentication)
_expected_token_hash = uhashlib.sha256(AUTH_KEY).digest()

# Global BLE UART object (created on boot)
ble_o = None

# Internal state for file transfer
_file = None        # file object for writing incoming data
_authorized = False # whether the connection is authenticated

def _on_rx(data):
    """
    Callback for handling data received over BLE.
    This function processes authentication tokens and file data.
    """
    global _authorized, _file

    try:
        if not _authorized:
            # Expecting an AUTH token in format b'AUTH:<token>'
            # Convert received bytes to string for easier parsing
            try:
                text = data.decode('utf-8')
            except Exception:
                text = ''
            if text.startswith("AUTH:"):
                token_str = text[5:]  # extract the token after "AUTH:"
                # Compute hash of the received token and compare with expected hash
                token_hash = uhashlib.sha256(token_str.encode('utf-8')).digest()
                if token_hash == _expected_token_hash:
                    _authorized = True
                    print("BLE: Authenticated successfully.")
                    # Open a temporary file for writing the new program
                    try:
                        _file = open("main.tmp", "wb")
                    except Exception as e:
                        ble_o.send("ERR:FS")  # filesystem error
                        print("BLE: Failed to open file for writing:", e)
                        _authorized = False
                    # Notify the central that auth is OK (ready to receive file)
                    if _authorized:
                        ble_o.send("AUTH_OK")
                else:
                    # Authentication failed
                    ble_o.send("AUTH_FAIL")
                    print("BLE: Authentication failed. Invalid token.")
                    # Remain not authorized (central can retry by sending token again)
            else:
                # Not authorized yet and received something that is not an auth token
                ble_o.send("ERR:AUTH")  # indicate error (not authenticated)
                print("BLE: Received data before auth. Ignoring.")
        else:
            # Already authorized, so this data should be part of the file transfer
            if _file is None:
                # No file open (should not happen if auth passed), ignore data
                return
            # Check for end-of-file marker: an empty message signals transfer completion
            if len(data) == 0:
                # End of file transfer
                _file.close()
                _file = None
                # Replace old main.py with the new file
                try:
                    import os
                    # Remove old main if exists, then rename temp to main.py
                    if "main.py" in os.listdir():
                        os.remove("main.py")
                    os.rename("main.tmp", "main.py")
                    ble_o.send("UPLOAD_DONE")
                    print("BLE: File transfer complete. Rebooting...")
                    # Small delay to allow notification to be sent before reset
                    import time, machine
                    time.sleep_ms(500)
                    machine.reset()
                except Exception as e:
                    ble_o.send("ERR:FS")
                    print("BLE: Error finalizing file transfer:", e)
                    # Remain in current state (do not reboot) so user can attempt again
                    _authorized = False
            else:
                # Write chunk to file
                try:
                    _file.write(data)
                except Exception as e:
                    ble_o.send("ERR:WRITE")
                    print("BLE: Write error:", e)
                    # Abort file transfer
                    try:
                        _file.close()
                    except:
                        pass
                    _file = None
                    _authorized = False
    except Exception as ex:
        # Catch-all for unexpected errors in the RX handler
        ble_o.send("ERR:EXCEPTION")
        print("BLE: Exception in _on_rx callback:", ex)
        _authorized = False
        if _file:
            try:
                _file.close()
            except:
                pass
            _file = None

# Instantiate the BLE UART service with the RX callback
ble_o = BLEUART(_ble, name= "YB-DEVICE", rx_callback=_on_rx)

def start():
    """
    Start advertising the BLE UART service. 
    Call this after initializing ble_o (called automatically in boot.py).
    """
    ble_o.advertise()
    print("BLE: Service started, awaiting connections...")

Explanation: This is the BLE manager module that ties everything together. It sets up the BLE UART service and defines how to handle incoming data from the BLE central (the browser). Key points:

- We initialize the BLE radio (_ble = bluetooth.BLE()) and create a global ble_o (BLE UART object) by calling BLEUART(_ble, ...). We give the device a name (in this example "YB-DEVICE", but you can customize it) which will appear when scanning for BLE devices. We also pass the _on_rx callback to handle received data.

- We pre-compute _expected_token_hash as the SHA-256 digest of the AUTH_KEY from settings. This is used to verify the token sent by the central. Using a secure hash means the actual key is not stored in plaintext in the device’s memory (a simple security measure)[2].

The _on_rx function is the heart of the file transfer protocol:
- If we are not yet authorized (_authorized is False), we expect the first message to be an authentication token. We decode the bytes to a string and check if it starts with "AUTH:". If so, we extract the token string and compute its SHA-256 hash, comparing it to the expected hash. If they match, we set _authorized = True and open a file main.tmp for writing the incoming program bytes. We then send an "AUTH_OK" notification back to the central to acknowledge that authentication succeeded and the device is ready to receive the file. If the token is wrong, we send "AUTH_FAIL" and remain unauthorized (the central may retry with a correct token). If we receive any data before authentication (that doesn’t start with "AUTH:"), we ignore it and optionally notify an error ("ERR:AUTH"). This handshake ensures that only an authorized user (who knows the token) can upload code.
- If we are authorized, then any incoming data is treated as part of the file content. We append each chunk of data to the open file. We also watch for a special end-of-transfer indicator: an empty BLE packet. In our protocol, when the web app finishes sending the file, it sends a zero-length packet to signal the end[5]. When _on_rx detects len(data) == 0, it concludes that file transmission is complete. At that point, the code closes the file and attempts to finalize the update: it deletes any existing main.py (if present) and renames the temporary file to main.py. If this is successful, it sends an "UPLOAD_DONE" confirmation to the central and then waits a brief moment (500 ms) before calling machine.reset() to reboot the microcontroller. On reboot, because a new main.py exists, the device will run that new user program. If any error occurs during this final step (e.g., file system error), it sends an error code ("ERR:FS") and does not reset – this allows the user to see the error and possibly retry.
- During file reception, if a write to flash fails (e.g., due to running out of space or other I/O error), _on_rx will catch the exception, send an error notification ("ERR:WRITE"), close the file, and reset _authorized to False (aborting the transfer). The central side should detect the error code and can choose to retry.
- A generic exception catch is included to handle any other unforeseen errors in the callback, sending "ERR:EXCEPTION" for debugging.

The ble.start() function simply calls ble_o.advertise() to (re)start advertising the BLE service. This is called at boot after setting up BLE, as you can see in boot.py. The advertising will make the device visible to the web interface. We print a message indicating the BLE service is started. (The device name in this example is "YB-DEVICE"; you should use a name that makes sense for your project or hardware, especially if multiple devices might be in range.)

Summary of BLE Protocol: The first message from the central must be "AUTH:<token>". If correct, the device responds with AUTH_OK. The central then sends the file data in 512-byte chunks (or smaller) sequentially. Once all file chunks are sent, the central sends an empty packet (""). The device then responds with UPLOAD_DONE and reboots itself. During the process, if any error occurs, the device sends an ERR:<code> message (and typically aborts the operation). The following codes are used in this implementation: - AUTH_OK – Authentication successful, ready to receive file. - AUTH_FAIL – Authentication failed (wrong token). - UPLOAD_DONE – File received and written, device will reboot. - ERR:AUTH – Data received out of order (was not authenticated yet). - ERR:FS – File system error (opening, renaming, etc.). - ERR:WRITE – Error writing data to flash. - ERR:EXCEPTION – Catch-all for other exceptions.

File: blerepl.py

# Placeholder for BLE REPL functionality (not implemented in this version)
# In the future, this could allow interactive REPL over BLE.
pass

Explanation: This is a stub for a future extension – providing a REPL (Read-Eval-Print Loop) over BLE, which could allow wireless interaction with the MicroPython interpreter. In the current project, this file does nothing (it just contains a pass so that importing it has no effect). It’s included to avoid import errors in boot.py and to indicate where such functionality could be added later.

File: main.py (User Program)

# main.py - User program (will be replaced by the uploaded file via BLE)

# This file can initially be empty or contain a simple demo.
# After you upload a new main.py using the web interface, that code will run.

# Example demo: blink the on-board LED (if any) or print a message periodically
import time
try:
    from machine import Pin
    led = Pin(2, Pin.OUT)  # using GPIO2 as example LED pin (adjust for your board)
except:
    led = None

print("main.py demo: running. Toggle LED every second.")
if led:
    led_state = False
else:
    # If no LED, just print instead
    led_state = None

while True:
    if led is not None:
        led_state = not led_state
        led.value(1 if led_state else 0)
    print("User program heartbeat...")
    time.sleep(1)

Explanation: This is a simple placeholder main.py to demonstrate that the user program is running. In a real scenario, this file will be overwritten by whatever script the user uploads via the web interface. The provided example toggles an LED on and off every second (assuming an LED is connected to GPIO2, which might be the case on some boards) and prints a heartbeat message. This is just to show that the device is operational. When you upload a new script using BLE, the device will reboot and run that new script instead of this one. (Note: If you prefer, you can leave the initial main.py essentially empty so that it doesn’t interfere or do anything until you upload a real program.)

File: main_ble_demo.py (Example Program to Upload)

# Example user program to test BLE upload (main_ble_demo.py)
# This program simply prints a message and flashes the RGB LED matrix in a pattern.

from yolobit import display, Image  # assuming yolobit library is available for LED matrix
import time

# Show a pattern on the 5x5 RGB matrix (a small dot moving in a square)
frames = [
    Image("90000:00000:00000:00000:00000"),  # dot at (0,0)
    Image("00000:00000:00000:00000:00900"),  # dot at (4,4)
    Image("00009:00000:00000:00000:00000"),  # dot at (0,4)
    Image("00000:00000:00000:00000:90000"),  # dot at (4,0)
]
print("BLE demo program: running. Press Button A to stop.")

# Loop through frames
while True:
    for img in frames:
        display.show(img)
        time.sleep(0.5)

Explanation: This is an example script that one could upload to the device to verify that the BLE update works. It uses the yolobit library (if available on the device) to control the on-board 5x5 LED display. It simply animates a moving dot and prints a message to the console. In practice, you can upload any Python script using the mechanism provided – this is just a sample. (If your board doesn’t have the yolobit library or the LED matrix, you can modify this demo to blink an LED or print messages as needed.)

Web Interface (BLE Upload Page)

The web interface is an HTML page with embedded JavaScript that uses the Web Bluetooth API to connect to the ESP32 and transfer the file. The interface will allow the user to select a .py file from their computer and send it to the device via BLE. Below is the code for the web page:

File: index.html (Web Bluetooth Uploader)

<!DOCTYPE html>
<html lang="en">
<head>
  <meta charset="UTF-8" />
  <meta name="viewport" content="width=device-width, initial-scale=1.0" />
  <title>ESP32 BLE Uploader</title>
  <style>
    body { font-family: Arial, sans-serif; margin: 2em; }
    h1 { font-size: 1.5em; }
    #logs { white-space: pre-wrap; background: #f9f9f9; padding: 1em; border: 1px solid #ccc; height: 150px; overflow-y: auto; }
    button, input[type="file"] { font-size: 1em; margin: 0.2em 0; }
    .hidden { display: none; }
  </style>
</head>
<body>
  <h1>ESP32-S3 BLE Firmware Uploader</h1>
  <p>This page allows you to upload a new <code>main.py</code> to your ESP32-S3 device over Bluetooth Low Energy.</p>
  <ol>
    <li>Put your device in BLE programming mode (ensure no user program is running or use the safe-mode button if needed).</li>
    <li>Click "Connect to Device" and pair with your ESP32 device (it should advertise as "YB-DEVICE" or your configured name).</li>
    <li>Select a <code>.py</code> file to upload, then click "Upload File".</li>
    <li>Wait for the transfer to complete and the device to reboot with the new program.</li>
  </ol>
  <button id="connectBtn">Connect to Device</button><br>
  <input type="file" id="fileInput" accept=".py" class="hidden" />
  <button id="uploadBtn" class="hidden">Upload File</button>
  <div id="logs"></div>

<script>
  const NUS_SERVICE_UUID   = "6e400001-b5a3-f393-e0a9-e50e24dcca9e";
  const NUS_CHAR_RX_UUID   = "6e400002-b5a3-f393-e0a9-e50e24dcca9e"; // write (Peripheral receives)
  const NUS_CHAR_TX_UUID   = "6e400003-b5a3-f393-e0a9-e50e24dcca9e"; // notify (Peripheral transmits)
  let bleDevice, nusService, rxChar, txChar;

  const logElem = document.getElementById('logs');
  function log(text, type="info") {
    console.log(text);
    logElem.textContent += text + "\n";
    logElem.scrollTop = logElem.scrollHeight;
  }

  // Enable the file input and upload button after connection
  const fileInput = document.getElementById('fileInput');
  const uploadBtn = document.getElementById('uploadBtn');

  // Handler for received notifications (device -> web)
  function handleNotifications(event) {
    const value = event.target.value;
    const textDecoder = new TextDecoder();
    let msg = "";
    try {
      msg = textDecoder.decode(value);
    } catch (e) {
      console.error(e);
    }
    if (msg) {
      log("Device: " + msg);
      if (msg === "AUTH_OK") {
        // Device ready to receive file
        sendFileChunks();
      } else if (msg === "UPLOAD_DONE") {
        log("Upload complete. Device will reboot.");
      } else if (msg.startsWith("ERR:")) {
        log("Error from device: " + msg, "error");
        // In case of error, you might handle retries or abort.
      }
    }
  }

  // Connect to the BLE device and setup the service/characteristics
  document.getElementById('connectBtn').onclick = async function() {
    logElem.textContent = "";  // clear log
    try {
      log("Requesting Bluetooth device...");
      bleDevice = await navigator.bluetooth.requestDevice({
        filters: [{ namePrefix: "YB-DEVICE" }],  // adjust to your device name prefix
        optionalServices: [ NUS_SERVICE_UUID ]   // specify UART service UUID so we can access it[7]
      });
      bleDevice.addEventListener('gattserverdisconnected', () => {
        log("Disconnected from device.");
        fileInput.classList.add('hidden');
        uploadBtn.classList.add('hidden');
      });
      log("Connecting to GATT server...");
      const server = await bleDevice.gatt.connect();
      log("Connected, getting UART service...");
      nusService = await server.getPrimaryService(NUS_SERVICE_UUID);
      log("Service found, getting characteristics...");
      rxChar = await nusService.getCharacteristic(NUS_CHAR_RX_UUID);
      txChar = await nusService.getCharacteristic(NUS_CHAR_TX_UUID);
      // Setup notifications from TX characteristic
      await txChar.startNotifications();
      txChar.addEventListener('characteristicvaluechanged', handleNotifications);
      log("UART service and characteristics ready.");
      // Enable file selection and upload UI
      fileInput.classList.remove('hidden');
      uploadBtn.classList.remove('hidden');
    } catch (error) {
      log("Connection failed: " + error, "error");
    }
  };

  // Read file and send it in chunks after sending auth token
  let fileData = null;
  fileInput.onchange = () => {
    if (fileInput.files.length > 0) {
      const file = fileInput.files[0];
      if (!file.name.endsWith(".py")) {
        alert("Please select a .py file.");
        fileInput.value = "";
        return;
      }
      // Read the file as text (assuming Python source is text)
      const reader = new FileReader();
      reader.onload = () => {
        fileData = reader.result;
        log(`Selected file "${file.name}" (${fileData.length} bytes). Ready to upload.`);
      };
      reader.readAsText(file);
    }
  };

  uploadBtn.onclick = async () => {
    if (!fileData) {
      alert("No file selected.");
      return;
    }
    if (!rxChar) {
      alert("Not connected to device.");
      return;
    }
    // Begin upload sequence
    log("Starting upload...");
    try {
      // 1. Send authentication token
      const token = prompt("Enter authentication token:");
      if (token === null) {
        log("Upload cancelled (no token provided).");
        return;
      }
      const authMessage = "AUTH:" + token;
      await rxChar.writeValue(new TextEncoder().encode(authMessage));
      log("Sent auth token, awaiting response...");
      // The device will respond with "AUTH_OK" or "AUTH_FAIL" via notifications.
      // When "AUTH_OK" is received, handleNotifications() will call sendFileChunks().
    } catch (error) {
      log("Upload failed: " + error, "error");
    }
  };

  async function sendFileChunks() {
    if (!fileData) return;
    const CHUNK_SIZE = 512;
    const encoder = new TextEncoder();
    let offset = 0;
    try {
      while (offset < fileData.length) {
        // Take a chunk of the file
        const slice = fileData.slice(offset, offset + CHUNK_SIZE);
        const chunkBuffer = encoder.encode(slice);
        // Write chunk to the RX characteristic
        await rxChar.writeValue(chunkBuffer);
        offset += CHUNK_SIZE;
        log(`Sent ${offset > fileData.length ? fileData.length : offset}/${fileData.length} bytes...`);
        // (Optional) small delay could be added here if needed to avoid overloading
      }
      // After sending all data, send an empty packet to signal end-of-file
      await rxChar.writeValue(new Uint8Array([]));
      log("All chunks sent, waiting for device to finalize...");
    } catch (error) {
      log("Error during file transfer: " + error, "error");
    }
  }
</script>
</body>
</html>

Explanation: This HTML/JS page provides a simple user interface and uses the Web Bluetooth API to communicate with the ESP32. Key parts of the implementation:

- Device Discovery and Connection: When the user clicks Connect to Device, the code calls navigator.bluetooth.requestDevice() with a filter for devices whose name starts with "YB-DEVICE" (you can adjust this to match the name set in your ble.py) and with the UART service UUID specified in optionalServices. The optionalServices is critical because we are filtering by name rather than service; including the UART service UUID allows the page to access that service after connecting[7]. The browser will display a BLE device chooser dialog. The user should select the ESP32 device. After selection, the script connects to the device’s GATT server and retrieves the primary service with UUID 6e400001-b5a3-f393-e0a9-e50e24dcca9e (NUS). Then it gets the RX and TX characteristics by UUID. It enables notifications on the TX characteristic (so the device can send data to the web app) and sets up an event listener handleNotifications to process incoming messages from the device. Upon successful connection and setup, the file input and upload button are revealed for use.
Note: Web Bluetooth requires a secure context (HTTPS or localhost) and a user gesture to initiate requestDevice[3][8], so the Connect button must be clicked by the user. Ensure you serve this page via HTTPS or as a local file through a server (not via the file:// protocol) – see the usage notes below.

- Logging: The page has a simple log area (<div id="logs">) where messages are appended. The log() function is used throughout to display status updates, errors, and device responses.

- Notification Handling: handleNotifications(event) is called whenever the device sends a notification on the TX characteristic. We decode the bytes to a string message. Based on the content, we perform actions:

- If the device sends "AUTH_OK", it means it accepted our token and is ready for the file. The handler then calls sendFileChunks() to start sending the file data.

- If "UPLOAD_DONE" is received, it indicates the device finished writing and is rebooting. We log this. (After reboot, the BLE connection will drop, triggering the gattserverdisconnected event we set up to log a disconnection.)

- If an error code like "ERR:..." comes, we log it as an error. Depending on the error, the user might decide to retry the upload. (The current script does not automatically retry on error; it simply logs it.)

- File Selection: When a file is chosen via the file input, the script reads it using a FileReader as text (assuming the Python file is text data, which is true for source code). The content is stored in fileData and a message is logged with the file name and size. We only accept .py files (set by the accept=".py" attribute and an extra check on file extension).

- Uploading Process: When the user clicks Upload File, the script first prompts for the authentication token (a simple prompt() dialog). The user must enter the same token that is set in the device (in setting.py). We then send the string "AUTH:<token>" via a BLE write to the RX characteristic. We log that the token is sent and then wait – the next steps depend on the device’s response. The device should reply with either AUTH_OK or AUTH_FAIL. If AUTH_FAIL or no response comes, the upload will not proceed (and an error would be logged). If AUTH_OK comes, our notification handler will call sendFileChunks().

- Sending File Chunks: The sendFileChunks() function takes the file content stored in fileData and sends it in increments of CHUNK_SIZE (512 bytes). It encodes each chunk to bytes and writes it via rxChar.writeValue(). It logs progress after each chunk. Once all chunks are sent, it sends an empty byte array as a final write to indicate the end-of-file to the device[5]. After that, it logs that all data is sent and waits for the device to finish (which should result in an UPLOAD_DONE notification and then a disconnection). We wrap the sending loop in a try/catch to handle any exceptions (e.g., if the device disconnects mid-transfer or BLE write fails). Optionally, we could add a small delay between chunk writes if needed to avoid saturating the BLE link – in practice, if the device’s BLE buffer is large enough (MicroPython default MTU is 20 bytes unless negotiated higher, but we use 512 as it will be split across multiple packets internally), this usually isn’t an issue. The Web Bluetooth API returns a Promise for writeValue which we await, ensuring each chunk is sent sequentially.

Important: Web Bluetooth will only work on secure contexts (HTTPS or localhost)[3]. If you open this HTML file directly as a local file, the API might be unavailable or scanning will not find devices. For testing, you can serve it via a simple local server (for example, using python -m http.server and accessing at http://localhost:8000, which is considered secure for Web Bluetooth). On some platforms (like Windows and Mac), no special flags are needed in Chrome, but on Linux or others you might need to enable experimental features (check Chrome’s about://flags if you run into trouble). Also, the name filter in requestDevice should match your device name. Here we used "YB-DEVICE" as the prefix, which corresponds to the name we set in ble.py. Change this if you set a different device name. Alternatively, you could filter by the service UUID instead, but using name is often more user-friendly if multiple BLE devices are around.

Usage Guide

Follow these steps to set up and use the ESP32-S3 BLE Firmware Uploader:

- Prepare the ESP32 Device: Flash the MicroPython firmware onto your ESP32-S3 if not already done. Then upload the provided MicroPython files (boot.py, setting.py, utility.py, bleuart.py, ble.py, blerepl.py, and an initial main.py) to the device’s filesystem. You can use tools like Thonny, ampy, or WebREPL to copy these files. Ensure that the files are in place and match the contents above. The AUTH_KEY in setting.py should be set to a secret token of your choice – remember this token, as you will need to enter it in the web interface. By default, it’s "my_secret_token" in the example; you should change it for security.

- Boot and Safe Mode: Power or reset the ESP32. If this is the first run (no main.py on the device), it will start in advertisement mode directly (you should see the LED matrix animation indicating it’s waiting for a program). If a user program is already present and running, and you want to reprogram the device, enter safe mode: hold down Button A on the device, then press reset (or power cycle). Continue holding Button A for ~2 seconds until you see the LED flash and messages about removing main.py. The device will delete the old main.py and reboot without it, so it will be idle and ready to accept a new upload. (This step is important if the currently running main.py either occupies the device or if you just want to replace it. The boot script’s safe mode is a safeguard against being “locked out” by a rogue program.)

- Open the Web Interface: Launch a web server to serve the index.html file (for example, run python3 -m http.server in the directory containing index.html, which will by default serve files at http://localhost:8000). Then open the page in Google Chrome (or a Chromium-based browser) by navigating to https://localhost:8000/index.html (if you have HTTPS set up) or http://127.0.0.1:8000/index.html. Note that Web Bluetooth requires a secure context – using http://localhost is allowed as a special case, or you can serve over HTTPS[3]. If you just open the file directly from your drive, the Web Bluetooth API will not work due to security restrictions.

- Connect to the Device via BLE: On the web page, click “Connect to Device”. The browser will ask for permission to scan for BLE devices. It should list your ESP32 device (look for the name you set, e.g., “YB-DEVICE”). Select it and click Connect. The page will then pair with the device and establish the GATT connection. You should see log messages appear in the “logs” area of the page as it finds the service and characteristics. If connection is successful, the “Upload File” button and file chooser will become visible. (If it fails, ensure that Bluetooth is enabled on your computer/phone, the device is powered and advertising, and that you are using a compatible browser. Chrome on Windows, Mac, Android, and Linux supports Web Bluetooth; Safari on iOS currently does not support Web Bluetooth as of 2025[9]. On some systems, you may need to enable the “Experimental Web Platform features” flag in Chrome’s settings if Web Bluetooth is not fully enabled.)

- Select the Python File: Click the file input field (or a “Choose file” button, depending on browser) and select the .py file you want to upload (for example, main_ble_demo.py or any other Python script). Once selected, the script will note the file name and size in the logs. Make sure the file size is within reason – transferring very large files over BLE may be slow (a few kilobytes should transfer quickly; large libraries of tens of kilobytes will take longer).

- Initiate File Upload: Click “Upload File”. The page will prompt you to enter the authentication token. Enter the same token that is configured on the device (the AUTH_KEY you set in setting.py). Then click OK. The web app will send the token to the device and wait for a response. In the logs, you should see “Sent auth token, awaiting response...”.

- If the token is correct, the device will respond with AUTH_OK, and the logs will show “Device: AUTH_OK”. The upload process will then automatically begin sending file chunks.

- If the token is incorrect, you’ll see “Device: AUTH_FAIL” in the logs. In that case, the device remains unauthenticated. You can try clicking “Upload File” again and entering the correct token. (If multiple failures occur, consider resetting the device – the device will continue advertising, and you can retry.)

- File Transfer: Once AUTH_OK is received, the script starts sending the file data. The log will update as chunks are sent (e.g., “Sent 512/2048 bytes...” etc.). This uses a 512-byte chunk size by design[4]. Under the hood, these chunks are being written to the BLE characteristic; the device buffers and writes them to flash. You may notice a small pause after each chunk – this is normal as the BLE stack processes the data. Do not navigate away or close the page during this process. If the file is small (a few KB), the upload will finish in a second or two. Larger files will take longer (roughly, BLE can transfer on the order of a few kilobytes per second in ideal conditions).

- Completion and Reboot: After the last chunk is sent, the web app sends an empty packet to signal the end of the file. The device then closes the file and replaces main.py. It will send a UPLOAD_DONE notification, which you’ll see in the logs as “Device: UPLOAD_DONE”. Shortly after, the device will reboot itself. In the logs, you might see the disconnection message (“Disconnected from device.”). The device will restart and execute the new main.py. Since it reboots into a user program, it will not immediately re-advertise the BLE UART (because now boot.py finds a main.py and doesn’t call ble.start() again). This is by design: once a user program is running, the BLE service stays running in the background if it was not explicitly stopped, but advertising is stopped because we’re connected, and after reboot we won’t advertise unless main.py or user action triggers it. If you want to upload another file, you should reset the device into safe mode again (or have the user program itself re-enable BLE advertising, though that would need additional code).

- Verify New Program: You can verify that the new main.py is running. If it’s a simple LED blink or print statement, observe the device (LED blinking, etc.) or connect to its serial console (via USB) to see printed output. For example, if you uploaded the provided main_ble_demo.py as main.py, the device’s 5x5 LED matrix should start showing a moving dot pattern and printing “BLE demo program: running...”. This confirms the upload succeeded.

- Troubleshooting:

  - If the web page can’t find the device when scanning, ensure the device is in advertising mode (no main.py running or in safe mode). Check that the name filter matches (you can also try removing the filter and using just acceptAllDevices: true with optionalServices, which will show all BLE devices – not as user-friendly, but good for debugging). Also confirm your browser supports Web Bluetooth and that you’re on HTTPS/localhost.

  - If the connection drops unexpectedly during upload, or you get an ERR:... message, there might be interference or the device might have reset early. You can press Connect again to reconnect (the device will advertise again after a disconnect) and then retry the upload. The safe mode button can be used if things get stuck.

  - For security, remember that while the token prevents unauthorized writes, BLE communication is not encrypted by default in this setup (no pairing/bonding used). An attacker sniffing BLE could see the token in plaintext if you use a simple token exchange. The use of SHA-256 in our code is mainly to avoid storing the plain token on the device and for integrity checking; it does not encrypt the traffic. For a more secure solution, consider implementing BLE pairing or a challenge-response authentication scheme. However, for most basic use (in a controlled environment), the token mechanism adds a layer of protection against unintentional overwrites.

- Extensibility: With the framework in place, you can extend this project by implementing blerepl.py to allow interactive REPL over BLE (perhaps triggered by a special command over the UART service). You could also integrate the web interface into a larger application or add features like progress bars for upload, file integrity verification (e.g., sending a hash of the file to verify it arrived correctly), and so on. The provided solution focuses on the core functionality of wireless code deployment.

Conclusion

You now have a complete system for updating an ESP32-S3’s MicroPython program over BLE using a web browser. This can be extremely useful for educational kits or IoT devices where frequent code changes are needed without physical access. The approach uses the standard Nordic UART Service for simplicity, and the Web Bluetooth API for broad accessibility (no custom native app required – a compatible browser is enough). Keep in mind the security and performance notes mentioned above. Happy coding wirelessly!

Sources:

- MicroPython official BLE UART example (Nordic UART Service implementation)[1][10] – provided inspiration for the BLEUART class and advertising payload structure.

- Stack Overflow discussion on sending files over Web Bluetooth[4][5] – informed the chunking strategy (512-byte chunks and an empty terminator packet).

- Chrome Developers documentation on Web Bluetooth[3][7] – clarified the need for secure context (HTTPS) and how to use name filters with optional services in requestDevice.

- MicroPython documentation on hashing algorithms[2] – confirms SHA-256 availability and preference over SHA-1 for secure purposes.

[1] [10] BLE UART - esp32 — Electronics for IoT

https://iot49.org/projects/bluetooth/ble_uart_esp32.html

[2] uhashlib – hashing algorithms — MicroPython 1.15 documentation

https://docs.micropython.org/en/v1.15/library/uhashlib.html

[3] [7] [8] Communicating with Bluetooth devices over JavaScript  |  Capabilities  |  Chrome for Developers

https://developer.chrome.com/docs/capabilities/bluetooth

[4] [5] javascript - Can I send files over web-bluetooth? - Stack Overflow

https://stackoverflow.com/questions/56469188/can-i-send-files-over-web-bluetooth

[6] raw.githubusercontent.com

https://raw.githubusercontent.com/micropython/micropython/master/examples/bluetooth/ble_advertising.py

[9] GitHub - Wave1art/ESP32-Web-Bluetooth: Simple example project using ESP32, Web Bluetooth

https://github.com/Wave1art/ESP32-Web-Bluetooth